DuroStrip V5 LED Lightbar - White

R•O•M DuroStrip V5 White LED compartment lights shall be supplied, lights shall be located on (left, right, or both) sides of each compartment door opening.  Lights shall be designed and manufactured to be waterproof, meeting the IP67 industry standard.  Lights shall have a streamline optic lens and produce a bright 367 Lumens per 12-inch section (nominal).  LED lights shall draw no more than 0.35 amps at 12.8vdc per 12-inch section of light strip.  LED lights shall be mounted in an anodized aluminum track provided by R•O•M either as a stand-alone unit or an integrated part of the roll up shutter door track.

LED lights shall be of flexible sealed design and shall be able to be replaced without the removal of the light extrusion assembly or shutter door.  LED lights shall be able to be integrated with any door switch control wired either negative switching or positive switching.

R•O•M DuroStrip V5 LED lights have a 7-year product warranty.

No Exceptions

DuroStrip V5 LED Lightbar - Red-White

R•O•M DuroStrip V5 Red-White LED compartment lights shall be supplied, lights shall be located on (left, right, or both) sides of each compartment door opening.  Lights shall be designed and manufactured to be waterproof, meeting the IP67 industry standard.  Lights shall have a streamline optic lens and produce 250 White Lumens and 60 Red Lumens per 12-inch section (nominal).  LED lights shall draw no more than 0.22 amps at 12.8vdc per 12-inch section of light strip.  LED lights shall be mounted in an anodized aluminum track provided by R•O•M either as a stand-alone unit or an integrated part of the roll up shutter door track.

LED lights shall be of flexible sealed design and shall be able to be replaced without the removal of the light extrusion assembly or shutter door.  LED lights shall be able to be integrated with any door switch control wired either negative switching or positive switching.

R•O•M DuroStrip V5 LED lights have a 7-year product warranty.

No Exceptions